# Testing Checklist - Abrome v1.1

Last updated: 9-4-2022 by Randy True

Direct any questions to support@floodlamp.bio

## Upon Arrival

- Water bath & amp heater turned on at least one hour before starting to process samples

- Can add boiling water from kettle to speed up the process

- Change water bath water weekly

## During Water Bath Heating

- Review freezer sensor data to ensure no thawing occurred.

- Prepare fresh 10% Bleach Solution, preferably daily, but at least every 3 days.

- Clean Inactivation Table thoroughly - first with 10% bleach, then with 70% ethanol or IPA.

- Set up Inactivation Table.

- Verify sufficient 1XISS is prepared (minimum 1 mL per sample, plus extra 5mL) AND < 2 weeks old

  - Otherwise prepare 1XISS: SOP-101-A_2 1X Inactivation Saline Solution Prep - Table Form - Abrome.pdf

- Verify sufficient aliquots of RM (one per sample + two for controls, plus extra) AND not expired (< 4 weeks old, no visible discoloration)

  - Otherwise prepare RM: SOP-102-A_2 Reaction Mix Prep - Table Form - Abrome.pdf

  - RM in strips-of-8 tubes to be used today can be kept in refrigerator until use

  - Any additional RM in strips-of-8 tubes should be placed immediately in freezer

    - Verify that caps are tightly sealed!

- Verify sufficient test consumables for usage (1 mL tips, 10 uL tips, gloves, wipes, aluminum foil)

- Intake collection tubes, preferably prior to the cutoff time, or during Amp

- Ideal would be to intake tubes as people return them, so as to address the uncollected tubes (tubes returned without the collection step completed)

- Ideally verify receipt of all expected collection tubes from students and staff

- Note any anomalies (e.g., uncollected tubes)

- Double check water bath is up to temperature (at least 95°C) before starting to process samples

## Running Test

- Turn off air purifiers before beginning the test procedure.  Keep the door closed.

- Tie back hair (if applicable).

- Run test:

  - Longer Training Amp Run Form:  SOP-104-A_1 Test Training - Run Form - Abrome.pdf

  - Standard Amp Run Form: SOP-103-A_1 Amplification - Run Form - Abrome.pdf

- Rerun test on any positive and inconclusive sample(s) in triplicate using the same inactivated sample(s) kept in the refrigerator.

- After running the test and any reruns, if any pools have returned positive or inconclusive results:

  - Communicate referrals to participants in those pools:

    - Surveillance Program Results Communications Guidelines.pdf

  - Enact your internal protocols accordingly.

## Cleanup

- Return cold blocks to the freezer.

- Return pipettes, tip boxes, racks.

- Check that all thawed inactivation solution bottles & vials are covered in foil.

- Discard sample tubes from refrigerator (neg. in trash, pos. in bag in bottom freezer).

- Turn off heaters.

- Empty tip buckets and clean them with bleach and ethanol.

# Weekly Checklist - Abrome - v1.1

Last updated: 9-2-2022 by Gary Withey

Direct any questions to support@floodlamp.bio

- Change water in water bath weekly.

- Preferable to use distilled water.

- Update complete Inventory according to schedule as agreed with FloodLAMP staff.

- Check Expiration dates of reagents.

- Empty trash bins.

- OPTIONALLY: Prepare reaction tubes for the week, and freeze.

# State Of Readiness Checklist - Abrome - v1.1

Last updated: 9-2-2022 by Gary Withey

Direct any questions to support@floodlamp.bio

The following should be in place at all times to ensure smooth operation of the lab:

- Glove boxes BOTH in assay prep area AND amplification area (keep separate glove supplies).

- Wipes present (one active, plus extra on supply shelf).

- Empty Tip box filled with strip tubes (fill tip box + half source box).

- Falcon Tubes Filled with strip tube caps (two full falcon tubes, + half source box).

- 1.5mL tubes closed and in cleaned, labeled cryobox (1 active box, Next box fully ready).

- At least 5 blank Amp run forms, and at least 1 spare of both table forms (1X ISS & RM).

- Pens and markers clean and in proper place.

  - Pens tested and functional.

- Spare tube of inactivation solution in freezer.

- Freezer thermometer tested and functional.

- Old non-significant samples clear from fridge.

- Tip buckets empty.