# Testing Checklist - Alpine v1.1

Last updated: 5-24-2022 RT GW BS

For all communication steps specified below, use the Slack channel:  # testing-alpine_portola

# Before Arrival

- Confirm Heaters turned on (text Randy & Theresa if confirmation not received by 8am)

# Upon Arrival      								Time:

- Prepare 10% Bleach Solution

- Double check water bath up to temp

- Clean Inactivation Table

- Set up Inactivation Table

- Check sufficient 1XISS (or make) and Rxn Mix (frozen or source tubes), and not expired

- Check sufficient consumables for usage (1.5mL tubes, strip-8s, tips, reservoirs)

- Ensure you have expected tubes from staff

- Check red mailbox for tubes  CODE: 513

# At Carillon

- Travel to Carillon and set up at drop off point           			Time:

- Intake Collection tubes already dropped off

- Continue intaking as participants drop off tubes until cut off time 9:05

- Communicate intake anomalies (QR code and anomaly description)

# Upon Return to Alpine						Time:

- Turn off air purifiers before beginning the test procedure.  Keep the door closed.

- Run test

- When on amp, communicate time expected to remove from amp

- Check with FL Program Admin if positives are known or unknown. Rerun if unknown.

- Communicate Result image & lookup map				Time:

  - Mark result on lookup map as a circled ‘P’ or ‘I’ at the end of the tube code

  - Wait 5 mins for confirmation of whether any positives are known positives

- Communicate re-run plan for all inconclusives and unknown positives

I.e.: “re-running 1 positive, 1 inconclusive in triplicate”

- Re-run test

- Communicate re-run results ETA

- Communicate final results and time					Time:

- Return cold blocks to freezer

- Return pipettes, tip boxes, racks

- Check that all thawed inactivation solution bottles & vials are covered in foil

- Discard sample tubes from refrigerator (neg. in trash, pos. in bag in bottom freezer)

- Turn off heaters

- Empty and clean tip buckets with bleach and ethanol

# Weekly Checklist - Alpine v1.1

- Update complete Inventory

- Submit inventory to FL

- Audit freezer sensor data for anomalies

- Check Expiration dates of reagents

- Empty trash bins

- OPTIONALLY: Prepare reaction tubes for the week, and freeze

- AS NEEDED:  receive deliveries

  - Use included ship list to verify contents

  - Communicate any discrepancies to # testing - alpine

  - Update Inventory with new materials

State Of Readiness Checklist - Alpine v1.1

- Glove boxes BOTH in assay prep room AND amplification area

- Wipes present (one active, plus extra on supply shelf)

- Empty Tip box filled with strip tubes (fill tip box + half source box)

- Falcon Tubes Filled with strip tube caps (two full falcon tubes, + half source box)

- 1.5mL tubes closed and in cleaned, labeled cryobox (1 active box, Next box fully ready)

- At least 5 blank Amp run forms, and at least 1 spare of both table forms (1X ISS & RM)

- Pens and markers clean and in proper place

  - Pens tested and functional

- Spare tube of inactivation solution in freezer

- Freezer thermometer tested and functional

- Lightbox battery charged and functional

- Old non-significant samples clear from fridge

- Tip buckets empty