/**

******************************************************************************

* @file    USART/Printf/main.c

* @author  MCD Application Team

* @version V3.3.0

* @date    04/16/2010

* @brief   Main program body

******************************************************************************

* @copy

*

* THE PRESENT FIRMWARE WHICH IS FOR GUIDANCE ONLY AIMS AT PROVIDING CUSTOMERS

* WITH CODING INFORMATION REGARDING THEIR PRODUCTS IN ORDER FOR THEM TO SAVE

* TIME. AS A RESULT, STMICROELECTRONICS SHALL NOT BE HELD LIABLE FOR ANY

* DIRECT, INDIRECT OR CONSEQUENTIAL DAMAGES WITH RESPECT TO ANY CLAIMS ARISING

* FROM THE CONTENT OF SUCH FIRMWARE AND/OR THE USE MADE BY CUSTOMERS OF THE

* CODING INFORMATION CONTAINED HEREIN IN CONNECTION WITH THEIR PRODUCTS.

*

* <h2><center>&copy; COPYRIGHT 2010 STMicroelectronics</center></h2>

*/

/* Includes ------------------------------------------------------------------*/

#include "stm32f10x.h"

#include "stm32_eval.h"

#include "flash.h"

#include "delay.h"

#include "ADS1256.h"

#include <stdio.h>

#include "ads1256_exti.h"

/** @addtogroup STM32F10x_StdPeriph_Examples

* @{

*/

/** @addtogroup USART_Printf

* @{

*/

/* Private typedef -----------------------------------------------------------*/

/* Private define ------------------------------------------------------------*/

/* Private macro -------------------------------------------------------------*/

/* Private variables ---------------------------------------------------------*/

/* Values magic to the Board keys */

#define  NOKEY  0

#define  KEY1   1

#define  KEY2   2

#define  KEY3   3

#define  KEY4   4

unsigned char data[16];

extern void ads1256_init(void);

// long ulResult;		//调试用

// long double ldVolutage;

/* Private function prototypes -----------------------------------------------*/

#ifdef __GNUC__

/* With GCC/RAISONANCE, small printf (option LD Linker->Libraries->Small printf

set to 'Yes') calls __io_putchar() */

#define PUTCHAR_PROTOTYPE int __io_putchar(int ch)

#else

#define PUTCHAR_PROTOTYPE int fputc(int ch, FILE *f)

#endif /* __GNUC__ */

/* Private functions ---------------------------------------------------------*/

void Init_UART2()

{

USART_InitTypeDef USART_InitStructure;

/* USARTx configured as follow:

- BaudRate = 115200 baud

- Word Length = 8 Bits

- One Stop Bit

- No parity

- Hardware flow control disabled (RTS and CTS signals)

- Receive and transmit enabled

*/

USART_InitStructure.USART_BaudRate = 9600;//波特率

USART_InitStructure.USART_WordLength = USART_WordLength_8b;//不需要校验，字长选择8位

USART_InitStructure.USART_StopBits = USART_StopBits_1;//停止位

USART_InitStructure.USART_Parity = USART_Parity_No;//校验位

USART_InitStructure.USART_HardwareFlowControl = USART_HardwareFlowControl_None;//硬件流控制，不使用

USART_InitStructure.USART_Mode = USART_Mode_Rx | USART_Mode_Tx;//串口模式：即发送又接收

STM_EVAL_COMInit(COM1, &USART_InitStructure);//完成串口COM1时钟配置、GPIO配置，根据上述参数初始化并使能

}

/**

* @brief  Main program

* @param  None

* @retval None

*/

int main(void)

{

unsigned char i=0;

long ulResult;

long double ldVolutage;//多精度浮点类型

ads1256_init();

Init_UART2();

Init_ADS1256_GPIO(); //初始化ADS1256 GPIO管脚

Delay(0x1ffFF);

GPIO_SetBits(GPIOB, GPIO_Pin_11 );

ADS1256_Init();

Reset_Trans_State ();

while(1)

{

//			ulResult = ADS_sum( ADS1256_MUXP_AIN0 | ADS1256_MUXN_AINCOM);

if(Get_Trans_State() == SET )    //读取数据传输状态，判断数据是否从ADS1256传输至单片机，传输完成则进行数据转换和打印

{

ulResult = Get_ulResult_exti();

if( ulResult & 0x800000 )    //判断采集数据是否为负数，如果是负数则进行补码转换，并把负数的原码赋值给ulResult

{														 //正数补码与原码相同不需要进行补码转换

ulResult = ~(unsigned long)ulResult;

ulResult &= 0x7fffff;

ulResult += 1;

ulResult = -ulResult;

}

ldVolutage = (long double)ulResult*0.59604644775390625;    //采集电压值，单位uV采集数据乘以5/2的23次方，采集数据转电压

ldVolutage = ldVolutage * 0.000001;    //采集的电压值，单位为V

ldVolutage = ldVolutage *(1000/5);    //（x/5）x为5V电压对应测量值即最大测量值  浊度1000   温度100，电压转测量值，后面可以加标定方程

printf("%.1lf",ldVolutage);    //double

printf("\n");

Reset_Trans_State ();    //传输完成标志位复位

GPIO_ResetBits(GPIOB, GPIO_Pin_12);    //片选引脚复位，进行新一轮的电压采集

}

}

}

/**

* @brief  Retargets the C library printf function to the USART.

* @param  None

* @retval None

*/

PUTCHAR_PROTOTYPE//发送一个字符协议，借助此函数测试串口是否正常

{

/* Place your implementation of fputc here */

/* e.g. write a character to the USART */

USART_SendData(EVAL_COM1, (uint8_t) ch);//发送函数，这是stm32库函数

/* Loop until the end of transmission */ //循环直到发送完成

while (USART_GetFlagStatus(EVAL_COM1, USART_FLAG_TC) == RESET)//如果发送未完成，标志位未置位，则循环等待

{}

return ch;

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t* file, uint32_t line)

{

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* Infinite loop */

while (1)

{

}

}

#endif

/**

* @}

*/

/**

* @}

*/

/******************* (C) COPYRIGHT 2010 STMicroelectronics *****END OF FILE****/