OVIS Carrier Board PCB Design Guide

| Connector Numbering | J61: 1~50; J62: 51~100 (J62 pin number in this document = physical pin number in BRD + 50) |
| --- | --- |
| Length Unit | mm |
| Scope | OVIS carrier board PCB component placement and routing |

General Rule Apply all differential-pair and length-matching requirements to the full link: existing main board traces + connectors/series components + carrier board traces. Length matching must account for the full link, not just the carrier board.

1. Key Requirements

Height clearance: Strict height limits apply beneath the main board footprint. Do not place tall components in this area until the allowable height is confirmed. Include component bodies, solder joints, and assembly tolerances in the height calculation.

Clock ground guarding: Use ground guards and a continuous GND reference for VI_CLK, SDIO0_CLK, SD1_CLK, and eMMC_CLK. Add nearby GND return vias at layer transitions.

Length matching: Complete direct routing first, then add length compensation near the receiver. Include existing main board trace lengths; do not evaluate only the local serpentine routing on the carrier board.

Differential pairs: Route P/N on the same layer with the same via count and structure, and maintain coupling. Do not cross reference-plane splits, slots, or board-edge cutouts.

Spacing: Maintain ≥3W spacing between high-speed signals, clocks, and switching nodes. Ground guards must not disturb differential impedance or create floating stubs.

2. Routing Constraints

| Interface | Impedance | Length Limit | Length-Matching Limit | Routing Requirements |
| --- | --- | --- | --- | --- |
| MIPI RX0~5 | 100 Ω differential | Full link ≤80 mm | P/N ≤0.30 mm; pair-to-pair ≤1 mm | Continuous GND; minimize layer changes; ground guards between groups |
| USB2 | 90 Ω ±10% | Full link ≤100 mm | P/N ≤0.20 mm | No more than 2 vias |
| EPHY RX/TX | Single-ended 50Ω | As short as possible | P/N ≤0.30 mm | Ground guards between P/N and between TX/RX |
| VI | 50Ω | As short as possible | ≤5 mm relative to VI_CLK | Ground-guard CLK; length-match DATA/SYNC |
| SDIO0 / SD1 | 50Ω | As short as possible | ≤5 mm relative to the respective CLK | Ground-guard CLK; length-match CMD/DATA |
| eMMC | 50Ω | Use the actual branch | Serpentine matching is usually unnecessary | Ground-guard the entire CLK trace; prefer an inner layer |

3. Differential-Pair Length Compensation

| Signal Group | P/N Pin | Main Board P (mm) | Main Board N (mm) | Main Board P-N (mm) | Carrier Board Target P-N (mm) | Full Link ¦P-N¦ (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| MIPI RX0 | J61-7 / J61-9 | 4.0104 | 3.5258 | +0.4846 | -0.4846 | ≤0.30 |
| MIPI RX1 | J61-14 / J61-12 | 1.9454 | 1.4473 | +0.4981 | -0.4981 | ≤0.30 |
| MIPI RX2 | J61-13 / J61-15 | 4.7916 | 4.4053 | +0.3863 | -0.3863 | ≤0.30 |
| MIPI RX3 | J61-18 / J61-20 | 1.5202 | 2.0057 | -0.4855 | +0.4855 | ≤0.30 |
| MIPI RX4 | J61-19 / J61-21 | 3.9352 | 4.2243 | -0.2891 | +0.2891 | ≤0.30 |
| MIPI RX5 | J61-24 / J61-26 | 2.0264 | 1.9818 | +0.0446 | -0.0446 | ≤0.30 |
| EPHY RX | J61-40 / J61-42 | 3.2269 | 3.1829 | +0.0440 | -0.0440 | ≤0.30 |
| EPHY TX | J61-44 / J61-46 | 3.1235 | 2.6553 | +0.4682 | -0.4682 | ≤0.30 |
| USB2 | J61-3 / J61-1 | 26.7305 | 26.7317 | -0.0012 | +0.0012 | ≤0.20 |

Carrier board target P-N = -(main board P-N). A positive value means P must be longer than N on the carrier board; a negative value means P must be shorter than N. If a connector or cable introduces P/N delay skew, subtract its equivalent length difference from this target.

| Interface | Connector Pin | Main Board Length | Maximum Available Carrier Board Length |
| --- | --- | --- | --- |
| MIPI RX0 | J61-7 / J61-9 | 3.7681 | ≤76.2319 - equivalent connector/cable length |
| MIPI RX1 | J61-14 / J61-12 | 1.6964 | ≤78.3037 - equivalent connector/cable length |
| MIPI RX2 | J61-13 / J61-15 | 4.5984 | ≤75.4016 - equivalent connector/cable length |
| MIPI RX3 | J61-18 / J61-20 | 1.7630 | ≤78.2370 - equivalent connector/cable length |
| MIPI RX4 | J61-19 / J61-21 | 4.0798 | ≤75.9202 - equivalent connector/cable length |
| MIPI RX5 | J61-24 / J61-26 | 2.0041 | ≤77.9959 - equivalent connector/cable length |
| USB2 | J61-3 / J61-1 | P 26.7305 / N 26.7317 | P≤73.2695 / N≤73.2683 - equivalent component length |

4. Single-Ended Length-Matching Groups

4.1 VI

| Signal | Connector Pin | Main Board Lmain (mm) | Main Board Offset from CLK (mm) | Carrier Board Compensation Target (mm) |
| --- | --- | --- | --- | --- |
| VI_CLK | J61-25 | 4.9228 | +0.0000 | -0.0000 |
| VI_D0 | J61-27 | 3.2921 | -1.6307 | +1.6307 |
| VI_D1 | J61-29 | 3.3535 | -1.5693 | +1.5693 |
| VI_D2 | J61-32 | 2.8003 | -2.1225 | +2.1225 |
| VI_D3 | J61-31 | 4.1692 | -0.7536 | +0.7536 |
| VI_D4 | J61-34 | 2.0264 | -2.8964 | +2.8964 |
| VI_D5 | J61-33 | 4.0652 | -0.8576 | +0.8576 |
| VI_D6 | J61-36 | 1.9228 | -3.0000 | +3.0000 |
| VI_D7 | J61-35 | 4.4174 | -0.5054 | +0.5054 |
| HSYNC | J61-37 | 3.5436 | -1.3792 | +1.3792 |
| VSYNC | J61-30 | 3.4496 | -1.4732 | +1.4732 |

Use VI_CLK as the reference. Carrier board compensation target = -(main board relative offset). After compensation, the full-link length mismatch within the group must be ≤5 mm.

4.2 SDIO0

| Signal | Connector Pin | Main Board Lmain (mm) | Main Board Offset from CLK (mm) | Carrier Board Compensation Target (mm) |
| --- | --- | --- | --- | --- |
| SDIO0_CLK | J62-82 | 16.9142 | +0.0000 | -0.0000 |
| SDIO0_CMD | J62-76 | 18.7224 | +1.8082 | -1.8082 |
| SDIO0_D0 | J62-86 | 17.0575 | +0.1433 | -0.1433 |
| SDIO0_D1 | J62-88 | 18.6471 | +1.7329 | -1.7329 |
| SDIO0_D2 | J62-90 | 15.7215 | -1.1927 | +1.1927 |
| SDIO0_D3 | J62-84 | 15.6427 | -1.2715 | +1.2715 |

Use SDIO0_CLK as the reference. Carrier board compensation target = -(main board relative offset). After compensation, the full-link length mismatch within the group must be ≤5 mm.

4.3 SD1

| Signal | Connector Pin | Main Board Lmain (mm) | Main Board Offset from CLK (mm) | Carrier Board Compensation Target (mm) |
| --- | --- | --- | --- | --- |
| SD1_CLK | J62-94 | 12.6758 | +0.0000 | -0.0000 |
| SD1_CMD | J62-98 | 14.6490 | +1.9732 | -1.9732 |
| SD1_D0 | J62-93 | 12.9477 | +0.2719 | -0.2719 |
| SD1_D1 | J62-95 | 13.2423 | +0.5665 | -0.5665 |
| SD1_D2 | J62-97 | 11.0696 | -1.6062 | +1.6062 |
| SD1_D3 | J62-99 | 12.2306 | -0.4452 | +0.4452 |

Use SD1_CLK as the reference. Carrier board compensation target = -(main board relative offset). After compensation, the full-link length mismatch within the group must be ≤5 mm.

4.4 eMMC Actual Branch

| Signal | Connector Pin | Actual Main Board Path | Lmain (mm) |
| --- | --- | --- | --- |
| EMMC_CLK | J62-61 | J62-61→R2438→U28 | 3.8936 |
| EMMC_CMD | J62-53 | J62-53→R2433→U28 | 3.7038 |
| EMMC_D0 | J62-55 | J62-55→R2434→U28 | 3.4099 |
| EMMC_D1 | J62-59 | J62-59→R2435→U28 | 3.6322 |
| EMMC_D2 | J62-63 | J62-63→R2436→U28 | 5.0784 |
| EMMC_D3 | J62-57 | J62-57→R2437→U28 | 5.1100 |

eMMC Note Calculate length along the full branch from J62 through R2433~R2438 to U28. Ground-guard the entire CLK trace and preferably route it on an inner layer.